# MANURE MANAGEMENT PLAN WORKBOOK

## CHECKLIST

|  | ### Manure Management Plan Page No. | ### Completed or Reviewed | ### Not Needed |
| --- | --- | --- | --- |
| # Section 1 – General Information |  |  |  |
| Date of Development | 2 |  |  |
| Contact Information | 2 |  |  |
| Operation Information | 3 |  |  |
| Animals Worksheet | 5 |  |  |
| # Section 2 – Mechanical Manure Application |  |  |  |
| Environmentally Sensitive Areas Worksheet | 6 |  |  |
| Winter Application Worksheet | 7 |  |  |
| Manure Management Plan Summary | 8 |  |  |
| # Section 3 – Operation Map |  |  |  |
| Operation Map | 9 |  |  |
| # Section 4 – Manure and Agricultural Process Wastewater Storage and Stockpiling/Stacking Areas |  |  |  |
| Agriculture Process Wastewater Worksheet | 10 |  |  |
| Manure Storage Worksheet | 11 |  |  |
| # Section 5 – Pasture Management |  |  |  |
| Pasture Management Worksheet | 12 |  |  |
| # Section 6 – Animal Concentration Areas |  |  |  |
| ACA Worksheet | 13 |  |  |
| Note: regulations require all operations with crops or ACAs to also have an Agricultural Erosion and Sediment Control Plan meeting the requirements of 25 Pa. Code Chapter 102. Additional information can be obtained from the county conservation district. |  |  |  |
| # Section 7 – Recordkeeping |  |  |  |
| Recordkeeping Forms | 14-17 |  |  |

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 1 – General Information

## DATE OF DEVELOPMENT

(See Workbook Instructions Page 3)

Date of Development:

Date of Update(s):

Note that the owner/operator is expected to evaluate the manure management plan annually and update the plan when necessary to keep the plan consistent with operation and manure management practices.

## CONTACT INFORMATION

(See Workbook Instructions Page 3)

Operation Name:

Name of Operator:

Name of Landowner(s):

Operation Physical Address:

City, State and Zip Code:

Operation Mailing Address:

City, State and Zip Code:

Phone Number	(Home/Barn):

(Cell):

Email Address:

## Manure Management Plan Preparer Contact Information

## (if other than owner/operator)

Preparer Name:

Preparer Organization:

Physical Address:

City, State and Zip Code:

Phone Number	(Business):

(Cell):

Email Address:

# MANURE MANAGEMENT PLAN WORKBOOK

## OPERATION INFORMATION

(See Workbook Instructions on Page 3)

| a. |  |  | Crop Rotations used on the Operation (use additional sheets as necessary): |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| b. |  |  | ###### Acres available for mechanical application of manure (excluding pasture): |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Owned: |  |  | Rented: |  |  |  |  |  |  |  |  |
|  |  | The term “Mechanical application” as used in this document means the application of manure by a person through any mechanical means such as a manure spreader, irrigation system, horse-drawn equipment, or a pitchfork. The term does not include direct application of manure by animals on pastures and/or in Animal Concentration Areas. Pasture Areas should only be noted once, in letter c. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c. |  | ###### Pasture Areas: |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | If yes, list acres: Owned: Rented: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | If the operation contains pasture, then complete the Pasture Management Worksheet. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d. |  | Total acres available for manure: (b. + c.) |  |  |  |  |  |  |  |  | Acres |  |  |  |
|  | e. |  | ###### Animals on the operation: |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | If animals are on the operation, complete the Animals Worksheet. Refer to Appendix 2, Agronomy Facts 54, Pennsylvania’s Nutrient Management Act (Act 38) for additional information. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | f. |  | ###### Environmentally Sensitive Areas: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Private or public drinking water wells |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | Streams, lakes, springs, or ponds |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | Open sinkholes |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | Areas of concentrated flow including swales, ditches, gullies, etc. |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | For winter application, above ground inlet to agricultural drainage system |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | If the operation contains any environmentally sensitive areas, then complete the Environmentally Sensitive Areas Worksheet and identify the environmentally sensitive areas on the Operation Map. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | g. |  | Winter Application: Is manure applied during the winter? |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | If yes, then complete the Winter Application Worksheet. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | h. |  | Agricultural Process Wastewater: Is any agricultural process wastewater generated on-site? |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | If yes, then complete the Agricultural Process Wastewater Worksheet. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | i. |  | Manure Storage Facilities: Is manure stored in a manure storage facility (concrete tank, metal tank, under-building structure, earthen, clay, or synthetic lined pond or lagoon, solid manure stacking pad, etc.)? |  |  | Yes |  |  | No |  |  |  |  |  |  |
|  |  |  | If yes, then complete the Manure Storage and Stacking Worksheet. |  |  |  |  |  |  |  |  |  |  |  |  |

| j. | ###### Solid Manure Stockpiling or Stacking: | Yes |  | No |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Is manure stockpiled or stacked in outdoor areas? |  |  |  |  |  |  |
|  | If yes, then complete the Manure Storage and Stacking Worksheet. |  |  |  |  |  |  |
| k. | ###### Animal Concentration Areas (ACAs): | Yes |  | No |  |  |  |
|  | If yes: Owned: Rented: |  |  |  |  |  |  |
|  | If the operation contains any ACAs, then complete the ACA Worksheet. |  |  |  |  |  |  |
| l. | Manure Spreader Calibration: The manure spreader used to apply all manure will be calibrated according to the recommendations in Agronomy Facts 68 found in Appendix 3 prior to manure application. | Yes |  |  |  | NA |  |

# MANURE MANAGEMENT PLAN WORKBOOK

## ANIMALS WORKSHEET

Use Additional Sheets as Necessary

(See Workbook Instructions on Page 4)

- Animal Unit and Animal Equivalent Unit Calculation:

| #### Animal Type #### (a) | #### Animal # (normal production day) #### (b) |  | #### Average Weight (lb.) #### (c) |  |  |  | #### Animal Unit (AU) #### (d) |  | #### Days on operation per year #### (e) |  |  |  |  | #### Animal Equivalent Unit (AEU) #### (f) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | × |  | ÷ | 1,000 | = |  | × |  | ÷ |  | 365 | = |  |
|  |  | × |  | ÷ | 1,000 | = |  | × |  | ÷ |  | 365 | = |  |
|  |  | × |  | ÷ | 1,000 | = |  | × |  | ÷ |  | 365 | = |  |
|  |  | × |  | ÷ | 1,000 | = |  | × |  | ÷ |  | 365 | = |  |
|  |  | × |  | ÷ | 1,000 | = |  | × |  | ÷ |  | 365 | = |  |
|  |  |  |  |  |  |  |  |  |  | Total |  |  |  |  |

Refer to page 14 and Agronomy Facts 54, Pennsylvania’s Nutrient Management Act (Act 38):  Who is Affected? found in Appendix 2 when completing the Animals Worksheet.

If the operation maintains greater than 2.00 AEU/acre and eight or more total AEUs according to Calculation 2, the operation is regulated under Pennsylvania’s Nutrient Management Act (Act 38), and the operator should consult with a certified nutrient management specialist before proceeding with the remainder of the Manure Management Plan.

- AEU/Acre Calculation:

Total AEUs (1f) = Total acres available for manure (Operation Information Page, Letter d) = Total AEUs ÷ Total Acres available for manure = AEUs/Acre AEUs ÷ Acres = AEU/Acre

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 2 – Mechanical Manure Application

## ENVIRONMENTALLY SENSITIVE AREAS WORKSHEET

Use Additional Sheets as Necessary

(See Workbook Instructions on Page 6)

All Environmentally Sensitive Areas listed should appear on the operation map as described in Section 3 of the Instructions.

| #### Field #### Identification | #### Environmentally Sensitive Area (stream, lake, pond, spring, open sinkhole, drinking water source, concentrated flow area) | #### Setback or restricted distance for mechanically applied manure |
| --- | --- | --- |

# MANURE MANAGEMENT PLAN WORKBOOK

## WINTER APPLICATION WORKSHEET

Use Additional Sheets as Necessary

(See Workbook Instructions Page 7)

| #### Field Identification | #### Type of Manure (Liquid or Solid, Animal Type) | #### Winter Season Application Rate | #### Percentage of Crop Residue | #### Type of Residue or Vegetative Cover | #### Field Slope Percentage |
| --- | --- | --- | --- | --- | --- |

# MANURE MANAGEMENT PLAN WORKBOOK

## MANURE MANAGEMENT PLAN SUMMARY

Use Additional Sheets as Necessary

(See Workbook Instructions Page 8)

| #### Crop Group and Yield (a) | #### Manure Group (b) | #### Application #### Season (c) | #### Planned #### Application #### Rate from #### C, NBS, PI * (d) | #### Incorporation #### Timing (e) | #### Commercial Fertilizer Application Rate (f) | #### Fields where this crop group can be used (g) |
| --- | --- | --- | --- | --- | --- | --- |

| Soil test results taken in the last three years indicate phosphorous levels (Mehlich 3-P levels) are less than 200 ppm. | Yes |  | No |  |
| --- | --- | --- | --- | --- |

If soil samples are over three years old or indicate phosphorous levels greater than or equal to 200 ppm, then base manure application rates on the phosphorous removal charts, a NBS calculated to phosphorous removal, or the Phosphorous Index developed by an authorized planner.

*C	-	The application rate was taken from the charts in Appendix 1.  Appendix 1 contains an explanation and example of how to use the rate charts when filling out this Manure Management Plan Summary.

NBS	-	The application rate was calculated using Nutrient Balance Sheet.

PI	-	The application rate was calculated by a Certified Nutrient Management Planner using the Phosphorus Index.

No single application can exceed 9,000 gallons unless applied in accordance with 25 Pa. Code § 83.294(e).  If any application rates are greater than 9,000 gallons, then split the application into multiple applications with no evidence of pooling between applications.

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 3 – Operation Map

## INSERT OPERATION MAP

(See Workbook Instructions Page 10)

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 4 – Manure and Agricultural Process Wastewater Storage and Stockpiling/Stacking Areas

## AGRICULTURAL PROCESS WASTEWATER WORKSHEET

Use Additional Sheets as Necessary

(See Workbook Instructions Page 11)

1.	Type of agricultural process wastewater generated on site (water system overflow, wash water, milkhouse wastewater, egg wash water, etc.)

| 2. Agricultural process wastewater is directed to a manure or waste storage facility listed on the Manure Storage and Stacking Worksheet and land applied according to recommendations in the Manure Management Plan Summary? | Yes |  | No |  |
| --- | --- | --- | --- | --- |
| a. If yes, identify the manure or waste storage facility or facilities listed on the Manure Storage and Stacking Worksheet that receive(s) the agricultural process wastewater. |  |  |  |  |
| b. If no, the operator should immediately contact the county conservation district, NRCS, or a private consultant for management recommendations and technical assistance. Identify the date, name, and affiliation of the contact in the space below. |  |  |  |  |
| Date of Contact Name and Affiliation of Contact |  |  |  |  |

i.	Description of management strategies for agricultural process wastewater generated on-site discussed with the county conservation district, NRCS, or private consultant.

ii.	Planned Implementation Date:

Implemented on Date:

# MANURE MANAGEMENT PLAN WORKBOOK

## MANURE STORAGE AND STACKING WORKSHEET

(Include Information for Each Manure Storage Facility and Stacking Area)

Use Additional Sheets as Necessary

(See Workbook Instructions Page 12)

1.	Type of storage(s) and stacking area(s) (concrete or metal tank, under building structure, earthen or clay or synthetically lined pond or lagoon, exposed concrete pad, roofed solid manure stacking pad, etc.) and year(s) of construction:

| - A copy of the professional engineer certification is kept on site for all liquid or semisolid manure storages constructed after January 29, 2000. | Yes |  |  |  |
| --- | --- | --- | --- | --- |
| - If a copy of the certification is not available, provide the date a registered professional engineer was contacted |  |  |  |  |

2.	Approximate size and volume of existing liquid and semisolid manure storages and/or the dimensions of existing stacking area(s), indicate if exposed to precipitation.

3.	Additional materials added to the manure storage(s) or stacking area(s) including bedding, silage leachate, and/or agricultural process wastewater (see the Agricultural Process Wastewater Worksheet):

| 4. The operation maintains adequate manure storage to apply manure according to the application recommendations outlined on the Manure Management Plan Summary. |  | Yes |  |  | No |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5. All manure stacking or stockpiling areas not on the farmstead meet the following criteria: |  |  | Yes |  |  | No |  |  |
|  | a. At least 100 feet from environmentally sensitive areas. b. On properly constructed and improved stacking areas whenever possible. c. On the top of a hill where possible, diverting upslope water away from the areas. d. On less than 8% slope. e. Manure is dry enough to stack at least four feet in height. f. The volume of stacked manure is limited to the amount that can be spread on near-by fields. g. Covered with a water-repellant cover if it will be in place for more than 120 days. |  |  |  |  |  |  |  |

6.	Actions or best management practices needed to address identified problems related to manure storage and/or stacking and the planned implementation date (season and year) for each practice or action:

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 5 – Pasture Management

## PASTURE MANAGEMENT WORKSHEET

(See Workbook Instructions Page 14)

List all pastures in the Manure Management Plan and identify these pastures on the operation map.

- Identify the pasture management approach below:

I am implementing grazing plan meeting the requirements of the Natural Resources Conservation Service Pennsylvania Technical Guide Practice Standard 528 for Prescribed Grazing.

I am managing or will manage my pastures by the date listed below to maintain at least three (3) inches of vegetation height and 70% perennial vegetative cover when animals are present on pasture.

Date Implemented or Planned Implementation Date

- If one of the above boxes are checked and animals are excluded from streams, seeps, ponds, and other surface waters, and clean drinking water is available to all livestock meeting their daily water requirements, identify the additional information below.

Exclusion Fence Length (ft)      		Average Width of Excluded Area (ft)

Installation Date of Fence and Watering System

If any fields are overgrazed, then they must be reestablished in the next growing season or those fields should be managed as an Animal Concentration Area (See the Animal Concentration Areas Worksheet).  Overgrazing means that the pasture is not meeting either of the pasture management guidelines identified in the above checkboxes.

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 6 – Animal Concentration Areas

## ANIMAL CONCENTRATION AREAS WORKSHEET

(See Workbook Instructions Page 15)

1.	Technical Assistance:

Some operations may need technical assistance to develop and implement BMPs on Animal Concentration Areas (ACAs) and/or develop a plan to minimize bare spots and maintain at least three inches of vegetation height and 70% perennial vegetative cover while animals are present on the pasture.  The operator has no more than three years from the date of developing the Manure Management Plan to implement BMPs or establish pasture conditions on ACAs.  DEP believes that most operations will be able to implement BMPs on a much shorter time frame but recognizes that more time may be needed for some costly BMPs.

Operators with ACAs requiring corrective actions should immediately contact the local conservation district, NRCS, or a private consultant and should document that contact and the time frame for developing and implementing BMPs.

List date contact was made to the assisting agency/party to help in these efforts:

List who was contacted to assist in these efforts:

2. Describe the management strategies for any ACAs on the operation.

3.	BMP Implementation Schedule

| Identify the date implemented in the “Date” row of the ACA block if BMP has been implemented. List the planned date for implementation in the “Date” row of the ACA block if the BMP is planned. Record N/A if the BMP does not apply. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| If installed, list the amount installed in the units listed in the “Amount” row of the ACA block. |  |  |  |  |  |  |  |
| #### ACA Name or Location (Refer to Operation Map) |  | #### Divert clean water around ACA #### (Number of Systems) | #### Improve and stabilize the surface material of the ACA #### (Sq. Ft) | #### Direct polluted water to storage or vegetated treatment area | #### Limit access to streams through stabilized crossings and watering areas | #### Limit size of denuded areas | #### Locate area where animals congregate (feed areas, shade, etc.) away from streams |
|  | Date |  |  |  |  |  |  |
|  | Amount |  |  |  |  |  |  |
|  | Date |  |  |  |  |  |  |
|  | Amount |  |  |  |  |  |  |
|  | Date |  |  |  |  |  |  |
|  | Amount |  |  |  |  |  |  |
|  | Date |  |  |  |  |  |  |
|  | Amount |  |  |  |  |  |  |

# MANURE MANAGEMENT PLAN WORKBOOK

# Section 7 – Recordkeeping Forms

## MANURE APPLICATION RATE RECORD

JANUARY 1,       THROUGH DECEMBER 31,

Use Additional Sheets as Necessary

(See Workbook Instructions Page 16)

| #### Date | #### Field Identification | #### Acres | #### Manure Group | #### Crop Group | #### Application Rate | #### Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ##### 10-2-22 | ##### 10, 12, 13 | ##### 3 | ##### Liquid Dairy | ##### Corn Silage | ##### 6,500 gal | ##### EXAMPLE |

# MANURE MANAGEMENT PLAN WORKBOOK

## CROP YIELD RECORD

JANUARY 1,       THROUGH DECEMBER 31,

Use Additional Sheets as Necessary

(See Workbook Instructions Page 16)

| #### Field Identification | #### Crop Group | #### Date Harvested | #### Yield Goal | #### Actual Yield Harvested | #### Notes |
| --- | --- | --- | --- | --- | --- |
| ##### 1, 3, 5, 7, 9 | ##### Corn Silage | ##### Sep 2022 | ##### 21 Tons | ##### 22 Tons | ##### EXAMPLE |

# MANURE MANAGEMENT PLAN WORKBOOK

## MANURE TRANSFER RECORD

JANUARY 1,       THROUGH DECEMBER 31,

Use Additional Sheets as Necessary

(See Workbook Instructions Page 16)

| #### Date | #### Name of Importer/Broker | #### Address and Phone Number Importer/Broker | #### Manure Group | #### Amount of Manure Transferred | #### Crop Group and Application Rate |
| --- | --- | --- | --- | --- | --- |
| ##### 4/20 | ##### EXAMPLE ##### Bill Jones | ##### 55 Manure Road Manure Town ##### 717-555-4567 | ##### Solid Beef | ##### 20 Tons | ##### Unknown |

# MANURE MANAGEMENT PLAN WORKBOOK

## MANURE STORAGE FACILITY RECORD

## MONTHLY INSPECTION FORM

Use Additional Sheets as Necessary

(See Workbook Instructions Page 16)

| #### Storage Name | #### Inspection Date | #### Manure Depth (liquid) | #### Depth of Surface of Manure to Freeboard (liquid) | #### Leak Detection System Inspections. Are there any leaks, overflows, or seepages? Describe. | #### Structural Integrity. Are there cracks, erosion, slope failures, liner deterioration, rodent holes, large vegetation, excessive or lush vegetation, fencing issues, loading area issues? Describe. |
| --- | --- | --- | --- | --- | --- |
| ##### EXAMPLE ##### Liquid Dairy | ##### 1/1/2022 | ##### 3.5 feet | ##### 7.5 feet | ##### None | ##### No problems observed |